# Pre-Lab 5: Inverting and Non-Inverting Amplifiers

# Part 1: Inverting Amplifier

  - Max supply voltage: +/- 22V

  - Max power consumption: 1000 mW

  - Input resistance: 1012 Ω

  - Input offset voltage: 5 mV max

  - Output Resistance: Low Impedance

  - Input offset current: 2 nA max

  - Voltage Gain: 106 dB

  - Pin 1: Balance

  - Pin 2: Inverting input

  - Pin 3: Non-inverting input

  - Pin 4: V-

  - Pin 5: Balance

  - Pin 6: Output

  - Pin 7: V+

  - Pin 8: NC

- The gain is -3 since it is -RfRi, the phase of the output will match the phase of the input exactly since the signal will be multiplied by -3/

- Since the voltage signal is being connected to the inverting input of the OpAmp. The significance of the resistors is their contribution to computing the gain, which is -RfRi

- You can Create an inverting buffer by connecting the voltage input to the inverting input of the OpAmp and then connecting the non-inverting to the output.

100 Ω

- Simulation for circuit 5.1

- As seen above, the orange line is the input, which ends up getting inverted, and multiplied by 3, so it has a gain of -3 due to the inverting amplifier, this output is seen for the blue line.

# Part 2: Non-Inverting Amplifier

- The closed-loop voltage gain of a non-inverting amplifier is 1+RfRi and the phase of the input will be seen on the output signal since the input signal is multiplied by the gain at each point.

- The circuit is called a non-inverting amplifier because the voltage input signal is connected to the non-inverting input of the OpAmp

- You can create a voltage follower by shorting the inverting input directly to the output.

- Circuit

Ri+Rf=10 V0.1  mA

Ri+Rf=100 kΩ

20=1+RfRi

Rf=19Ri

20Ri=100 kΩ

Ri=5 kΩ→Rf=95 kΩ

- Simulation

- Orange is input, and blue is output. The input is scaled by 3.