# Lab Report 6: OP-amp Circuit Analysis

# Integrator and Differentiator Circuit Behavior

The circuit is an inverting integrator that transforms the input 1 kHz square wave into a triangle wave output, confirming the mathematical operation of time-integration where a constant voltage results in a linear ramp. The experimental results align with theoretical predictions, showing an inverting phase relationship where the output ramps downward during the positive half-cycle of the input. While an ideal integrator would produce perfectly linear slopes, the inclusion of the 100 kΩ feedback resistor R2 for DC stability introduces the slight exponential curvature observed in the simulation and scope captures. This measured behavior successfully verifies the integrating functionality of the LF356 op-amp configuration.

The inverting differentiator circuit successfully converts the input triangle wave into a square wave output, confirming the mathematical operation where the constant slope of a ramp results in a steady-state DC voltage. The experimental and simulation results align with theoretical predictions, showing an inverting relationship where the output jumps to a negative constant during the triangle wave's upward (positive) slope and a positive constant during the downward slope. The "spikes" or ringing observed at the transition points in both the scope and simulation are characteristic of the circuit's high-pass filter behavior, as the sudden change in slope at the triangle wave's peaks represents a high-frequency component that is heavily amplified. This measured behavior—transforming a linear ramp into a horizontal step with a 90° phase lead—effectively verifies the differentiating functionality of the op-amp configuration.

At low frequencies (below ~100 Hz), the gain is flat at approximately 0 dB, showing that the feedback resistor R2 successfully limits the DC gain to prevent saturation Av=-R2/R1=-100k/100k=1. As the frequency increases, the curve hits its cutoff frequency fc at roughly 338 Hz, where the gain begins to roll off at a constant rate of -20 dB/decade. This linear downward slope on the logarithmic scale represents the "integration region," where the circuit's impedance is dominated by the capacitor, effectively attenuating higher-frequency components. The "null" and subsequent rise seen around 1 MHz is likely due to the gain-bandwidth product (GBW) limitations and parasitic effects of the LF356 op-amp itself, indicating that the circuit loses its ideal integrating properties at extremely high frequencies.

The Bode plot illustrates that the differentiator acts as a high-pass filter within its operational range, but with a critical stability limit. At frequencies below approximately , the gain increases at a constant rate of , which is the signature of a differentiator as it becomes more reactive to faster signal changes. The sharp peak at roughly represents the circuit's resonant frequency, where the open-loop gain of the LF356 op-amp meets the rising closed-loop gain, often leading to the ringing and instability seen in the previous time-domain captures. Beyond this peak, the gain rolls off steeply, indicating that the op-amp can no longer track the signal, effectively acting as a low-pass filter for high-frequency noise. This frequency response explains why the circuit excels at detecting the sharp transitions (high-frequency components) of the triangle wave but is also prone to the high-frequency oscillation (the "spikes") observed on the oscilloscope.